Lemnisca Scale-Up Risk Predictor — Mathematical Reference

All derivations, correlations, constants, and thresholds in the active calculation engine. Red boxes flag assumptions or values that should be reviewed.

0. Pipeline overview

The application executes the following sequence on every assessment:

InputForm collects ProcessInputs with field-level validation and warnings.

runAllDerivations(inputs) computes D1–D7 derived parameters.

buildReactorScaleConfigs(inputs, {method}) applies the chosen scale-up criterion and builds ReactorScaleConfig objects for lab and target. kLa, gassed power, and tip speed are computed here, not as separate derivation steps.

Risk modules R1–R5 run in fixed order: OTR → Mixing → Shear → CO₂ → Heat. Each consumes derived parameters and both reactor configs.

determinePrimaryBottleneck picks the highest severity domain; ties broken by a normalised "risk pressure" metric.

What-if analysis modifies ProcessInputs (impeller, RPM, VVM, O₂, dt_ratio_target, or target_rpm_override) and re-runs runAssessment.

⚠  Lab power and lab kLa are derived twice — once in D3 (derivePowerInput) and again inside buildReactorScaleConfigs. Risk modules consume the reactor-config values; the D3 outputs are used only for surfacing validation flags.

1. Input form and guardrails

Input form enforces field-by-field ranges. Representative bounds:

Lab volume V_lab: > 0, ≤ 1000 L. Target volume V_target: > V_lab.

H/D ratio: 0.5–4.0. Default 1.2.

RPM: > 0, ≤ 3000. VVM: 0.1–5.0 (default 1.0).

Biomass: > 0, ≤ 200 g/L CDW (or ≤ 100 OD600 with organism-dependent conversion).

Temperature: clamped to viscosity / C* table range (20–42 °C).

DO setpoint: percent saturation (0–100).

VVM outside 0.5–2.0 raises a power-correlation uncertainty flag (derivePowerFlags).

Computed lab P/V outside 500–8000 W/m³ across all three what-if scenarios raises an atypical-envelope flag.

2. Derivations D1–D7

D7 — Biomass CDW conversion (runs first, feeds D1)

CDW = OD600 × f_organism       if input unit is OD600

CDW = biomass                  if input unit is g/L

OD-to-CDW conversion factors (g/L per OD unit) per organism:

E. coli 0.37 · B. subtilis 0.36 · S. cerevisiae 0.40 · P. pastoris 0.38 · generic 0.38

Ref: Standard textbook conversions; see Doran (2013) Bioprocess Engineering Principles, 2nd ed., Ch. 8.

D1 — Oxygen uptake rate (OUR)

OUR is either user-measured or estimated from organism-specific peak bounds keyed on a low-density / high-density biomass category.

OUR_peak = upper_bound   if biomass_density_category = high_density

OUR_peak = lower_bound   otherwise

Bounds (mmol O₂ / L / h):

E. coli 70–150  ·  B. subtilis 35–80  ·  S. cerevisiae 20–55  ·  P. pastoris 25–160

High-density threshold: CDW ≥ 60 g/L. Representative CDW for each category: 20 g/L (low-density), 100 g/L (high-density).

Ref: Bounds compiled from species reviews: Shiloach & Fass (2005) Biotechnol Adv 23:345; Looser et al. (2015) Biotechnol Adv 33:1177 (P. pastoris); Sonnleitner & Käppeli (1986) Biotechnol Bioeng 28:927 (S. cerevisiae).

⚠  The estimation collapses peak OUR to a binary low/high based on a 60 g/L threshold. Real OUR varies continuously with biomass, nutrient profile, and feeding strategy. Encourage users to provide measured OUR whenever available.

D2 — Vessel geometry (lab and target)

V (m³)      = V_litres / 1000

T (tank D)  = ((4·V) / (π·H/D))^(1/3)

H_L         = (H/D) · T

d_imp       = (d/T)_input · T            d/T default per impeller: see §6

A_cross     = π · T² / 4

D3 — Power input (single + total, gassed)

Ungassed power for one impeller:

P₀ = N_p · ρ · N³ · d_imp⁵         (W)

Power numbers (N_p): Rushton 5.0 · Pitched-blade 1.5 · Marine 0.35 · Unknown 5.0.

Ref: Bates, Fondy & Corpstein (1963); Nienow (1990) Mixing in the Process Industries.

Gassed power via correlation by impeller type:

Rushton/Smith:    Pg = 0.706 · (P₀² · N · d_imp³ / Q_g^0.56)^0.45   [Michel & Miller 1962]

Pitched-blade:    Pg = 0.648 · (P₀² · N · d_imp³ / Q_g^0.56)^0.45   [Cui et al. 1996]

Hydrofoil/marine: Pg/P₀ = clamp(1 − 12·Fl, 0.30, 1.00),  Fl = Q_g/(N·d_imp³)

For multi-impeller systems:

Pg_total = n_imp · Pg_single · interaction_factor

interaction_factor = min(1.0, 0.85 + 0.15 · spacing_ratio)

⚠  Bug in correlations/gassed_power.ts (line 61): spacing_ratio is hard-coded as D_T/D_T = 1.0, so the interaction factor is always 1.0. Multi-impeller power is linearly scaled by n_imp with no real penalty for closely spaced impellers. Real systems lose 10–20% per additional impeller for spacing < 1·D.

Ref: Michel & Miller (1962) AIChE J 8:262; Cui et al. (1996) Biotechnol Bioeng 49:456; Nienow (2006) Chem Eng Sci 61:2925.

D-Viscosity — broth viscosity from temperature

Linearly interpolated from a water-like table (Pa·s):

20 °C 1.00e-3  ·  25 °C 8.9e-4  ·  30 °C 8.0e-4  ·  37 °C 6.9e-4  ·  42 °C 6.2e-4

For broths above the non-Newtonian biomass threshold (CDW > 60 g/L) the apparent viscosity (cP) is computed from a quadratic in (CDW − 60):

μ_app (cP) = 1.25e-3 · ΔX² + 0.175 · ΔX + 1     ΔX = max(0, CDW − 60)

Calibrated to: 1 cP at 60 g/L, 10 cP at 100 g/L, 50 cP at 200 g/L.

⚠  The non-Newtonian viscosity model is an empirical quadratic calibrated to three points. It is not a literature correlation. Real broth rheology varies by an order of magnitude between bacterial, yeast, and filamentous-fungus morphologies and between vegetative and stationary phase. Treat μ_app outputs as order-of-magnitude.

D4 — Impeller Reynolds number

Re_imp = ρ · N · d_imp² / μ

Regime split: turbulent if Re > 10⁴; transitional if 10³ ≤ Re ≤ 10⁴; laminar otherwise. Transitional regime raises a flag ("power number assumption may not hold"); laminar raises a hard warning.

D5 — Superficial gas velocity

Q_gas (m³/s) = VVM · V_litres / (1000 · 60)

v_s (m/s)    = Q_gas / A_cross

D6 — Oxygen solubility and driving force (lab + target)

Saturation C* is interpolated linearly from a temperature table (mmol/L at 1 atm air):

20 °C 0.276 · 25 °C 0.258 · 30 °C 0.237 · 35 °C 0.213 · 37 °C 0.204 · 42 °C 0.186

Pressure and inlet-O₂ corrections at lab and target:

P_top = P_atm

P_bot = P_atm + ρ · g · H_L

y_scale = (O₂_inlet / 100) / 0.209          0.209 = O₂ in dry air

C*_bot = C*_table · (P_bot/P_atm) · y_scale

C*_top = C*_table · (P_top/P_atm) · y_scale

C_L     = (DO_setpoint / 100) · (C*_bot + C*_top)/2

Log-mean driving force across vessel height (falls back to arithmetic mean when the two end values nearly coincide, and to bot-only when the top can't transfer):

ΔC_lm = (ΔC_bot − ΔC_top) / ln(ΔC_bot / ΔC_top)

Ref: Solubility table from Geankoplis (2003) Transport Processes and Separation Process Principles, 4th ed., Table A.3-18. Hydrostatic pressure correction follows Doran (2013) §10.7.

⚠  Bubble-side O₂ depletion is ignored — the model assumes the inlet O₂ fraction persists from sparger to surface. This overestimates kLa-effective driving force by ~10–15% in high-density runs (Tier-3 correction not implemented per design decision).

3. Scale-up criteria

A scale-up criterion fixes one operating variable as identical between lab and target, and solves for the target RPM (and sometimes VVM) by bisection on the gassed-power expression in §2 D3.

3.1 Constant P/V (default)

Holds gassed power per volume Pg/V constant.

(Pg/V)_target = (Pg/V)_lab

Solve for N_target such that gassedPower(geom_target, N_target, VVM_lab) / V_target = (Pg/V)_lab

Bisection uses 80 iterations, doubling the upper bracket until the target power is achievable. The result is clamped to a scale-dependent max RPM and max P/V (see §4); when clamped, a flag is raised and the criterion is reported as "not fully matched".

Ref: Constant P/V is the historical Merck/Pfizer default; Junker (2004) J Biosci Bioeng 97:347.

3.2 Constant kLa

Holds the kLa ensemble mean constant. VVM at target is set to the scale's maximum VVM (more aeration is favoured at large scale because P/V tends to drop).

kLa_target = kLa_lab

Solve N_target such that computeKlaEnsemble(...).mean = kLa_lab

Used when oxygen transfer is the governing constraint and P/V is a poor proxy across geometries.

3.3 Constant tip speed (shear-preserving)

v_tip = π · N · d_imp = constant

N_target = (v_tip_lab) / (π · d_imp_target) · 60     (RPM)

Used for shear-sensitive organisms (filamentous fungi, mammalian cells, fragile morphologies). Trades off mixing intensity and oxygen transfer to limit cell damage.

Ref: Tip-speed-based scaling: Oldshue (1966) Biotechnol Bioeng 8:3; Marks (2003) Cytotechnology 42:21.

4. Scale-dependent operating envelopes

Before the criterion result is reported, target RPM, P/V, and VVM are clamped to literature-supported ranges by volume bin (Math.sqrt-of-decade boundaries):

1 L    : P/V 3000–8000 W/m³ · RPM 800–1500 · VVM 1.0–2.0

10 L   : P/V 2500–6000 W/m³ · RPM 400–900  · VVM 0.8–1.5

100 L  : P/V 1500–4000 W/m³ · RPM 200–500  · VVM 0.6–1.2

1000 L : P/V 800–3000 W/m³  · RPM 100–250  · VVM 0.5–1.0

10000+L: P/V 500–2000 W/m³  · RPM 50–150   · VVM 0.3–1.0

Ref: Envelopes per user-provided engineering ranges (2026-05-07); broadly consistent with Junker (2004) and Doran (2013) Table 10.4.

5. kLa correlation ensemble

computeKlaEnsemble selects 2 correlations based on broth rheology:

Newtonian       (CDW ≤ 60 g/L): linek_2004,         vant_riet_non_coalescing

Non-Newtonian   (CDW > 60 g/L): garcia_ochoa_2004,  vant_riet_non_coalescing

The reported kLa is the arithmetic mean of the two correlations; std, min, max are also exposed for confidence-band display. All correlations take Pg in W and return kLa in s⁻¹ (converted ×3600 to h⁻¹ for display).

Active correlations

van't Riet non-coalescing : kLa = 0.002  · (Pg/V)^0.7  · v_s^0.2

Linek (2004) Rushton      : kLa = 0.0083 · (Pg/V)^0.62 · v_s^0.49

Linek (2004) Pitched-blade: kLa = 0.0063 · (Pg/V)^0.63 · v_s^0.58

Linek (2004) hydrofoil/other: kLa = 0.0057 · (Pg/V)^0.67 · v_s^0.56

Garcia-Ochoa & Gomez 2004 : kLa = 0.015  · (Pg/V)^0.6  · v_s^0.6  · (μ_w/μ_eff)^0.5

Ref: van't Riet (1979) Ind Eng Chem Process Des Dev 18:357; Linek et al. (2004) Chem Eng Sci 59:5145; Garcia-Ochoa & Gomez (2004) Biotechnol Adv 22:495.

Inactive correlations (in registry, never selected)

van't Riet coalescing · Linek 1987 · Moucha 2003 · Zhu 2001 (non-Newtonian)

⚠  The Zhu (2001) and Moucha (2003) correlations are defined but never selected by selectKlaEnsemble. For genuinely non-Newtonian broths (CMC, pellet morphology) Zhu's μ_app^(-0.84) dependence would be more appropriate than Garcia-Ochoa's μ^(-0.5).

⚠  Multi-impeller systems use single-impeller kLa correlations on the total Pg/V. This is intentional per the selection logic comment, but Moucha (2003) gives n-impeller-specific coefficients that would be more accurate for n_imp ≥ 2.

⚠  Linek 2004 coefficients for hydrofoil/marine impellers (0.0057, 0.67, 0.56) are the paper's hydrofoil values; the original measurement set is small and extrapolation to generic "marine" or "unknown" impellers is approximate.

6. R1 — Oxygen transfer risk

Compares achievable OTR against required OUR.

OTR_capacity = kLa · ΔC_lm        (mmol/L/h)

ratio        = OTR_capacity / OUR_peak

Score thresholds on the ratio:

low > 1.5  ·  moderate > 1.0  ·  high > 0.7  ·  critical otherwise

Per-scale evaluation: lab uses inputs.rpm / inputs.vvm directly; target uses criterion-derived RPM/VVM. Conservative/moderate/aggressive scenarios scale target Pg by 0.5×/1.0×/2.0× to show robustness to operating-point uncertainty.

Ref: Bands chosen to align with Doran (2013) §10.10: ratio<1 → guaranteed limitation; ratio 1.0–1.5 → marginal; ratio>1.5 → adequate buffer for non-idealities.

⚠  Score is taken from the moderate scenario (1.0× Pg) only. The conservative and aggressive numbers are shown but do not affect the headline score.

7. R2 — Mixing risk

Compares the time the dissolved-O₂ reservoir at C_L can sustain local OUR against the time to circulate the vessel.

θ_mix = 5.9 · T² / (ε^(1/3) · d_imp^(4/3))       (Ruszkowski 1994)

ε     = Pg/V / ρ          (specific energy dissipation, W/kg)

t_O2  = 3600 · C_L / OUR_peak                    (s)

margin = t_O2 / θ_mix

Score thresholds:

low > 10  ·  moderate > 1.0  ·  high > 0.1  ·  critical otherwise

Bands reflect that gradients dissipate before they form when t_O2 >> θ_mix, while t_O2 < θ_mix means a circulating fluid packet depletes its dissolved O₂ before the next encounter with the impeller — leading to oscillating DO and overflow metabolism.

Ref: Ruszkowski (1994) 8th Eur Conf Mixing: θ_mix correlation for turbulent single-phase stirred tanks. Bands consistent with Larsson & Enfors (1996) Biotechnol Bioeng 51:42 and Bylund et al. (1998) Biotechnol Bioeng 60:589 (E. coli scale-down studies).

⚠  Ruszkowski is validated for H/D 0.8–1.5 and single-impeller systems. For H/D > 1.5 a flag is raised; multi-impeller circulation patterns can shorten θ_mix by 30–50%.

8. R3 — Shear stress risk

Tip speed measured against an organism-specific tolerance.

v_tip = π · N · d_imp        (m/s)

ratio = v_tip / v_threshold

margin = v_threshold / v_tip

Organism tolerances (m/s):

E. coli 8.0 · B. subtilis 7.0 · S. cerevisiae 5.0 · P. pastoris 4.0 · generic bacteria 7.0 · generic yeast 5.0

Score thresholds on the ratio (note: higher ratio = more shear = worse):

low < 0.7  ·  moderate < 1.0  ·  high < 1.3  ·  critical otherwise

Tip speed is the controlling variable because Kolmogorov microscale at typical industrial dissipation is set by conditions in the impeller swept zone, where v_tip is the dominant velocity scale.

Ref: Tolerance values are mid-range engineering practice; primary literature varies widely. E. coli: Hewitt et al. (1998) Biotechnol Bioeng 60:744 reports tolerance to v_tip > 10 m/s. Yeast: Bagshaw (1992) Biotechnol Bioeng 39:1.

⚠  Tip-speed thresholds are indicative ranges, not hard physiological limits. E. coli can routinely tolerate 10–12 m/s under good DO control. Conservative values are used here to flag risk early.

9. R4 — CO₂ accumulation risk

Activated when CDW > 20 g/L or OUR > 30 mmol/L/h; otherwise score is forced to low.

Carbon dioxide evolution rate:

CER = RQ · OUR_peak           (mmol CO₂ / L / h)

RQ defaults: bacteria & S. cerevisiae 1.00 · P. pastoris (methanol) 0.67.

Outlet gas mole fraction (capped at 0.20 for numerical sanity):

ṅ_gas = Q_gas (NL/h) / 22.4 (L/mol)                   (mol/h)

y_CO2_out = min(4e-4 + (CER · V_L) / ṅ_gas , 0.20)

Log-mean gas-phase pCO₂ (inlet 4×10⁻⁴ × 1.01325 bar, outlet × mid-column pressure):

P_mid_bar  = (P_atm + ρ·g·H_L/2) / 1e5

pCO₂_gas_out = y_CO2_out · P_mid_bar

pCO₂_gas_avg = logMean(pCO₂_gas_out, pCO₂_gas_in)

Bulk dissolved pCO₂ adds Henry's-law driving force needed to strip CER:

pCO₂_bulk_atm = pCO₂_gas_avg/1.01325 + (CER/1000) / (kLa_CO2 · H_CO2)

kLa_CO2       = 0.9 · kLa_O2          H_CO2 = 0.034 mol/L/atm at ~30 °C

Hydrostatic correction (worst case at vessel bottom):

pCO₂_bottom = pCO₂_bulk · (P_atm + ρ·g·H_L) / P_atm

margin      = pCO₂_critical / pCO₂_bottom

Critical pCO₂ per organism (bar):

E. coli 0.20 · B. subtilis 0.15 · S. cerevisiae 0.10 · P. pastoris 0.15 · generic bacteria 0.15 · generic yeast 0.10

Score thresholds on the margin:

low > 1.5 · moderate > 1.0 · high > 0.75 · critical otherwise

Ref: CO₂ mass balance: Royce & Thornhill (1991) AIChE J 37:1680. Critical pCO₂ thresholds: Mostafa & Gu (2003) Biotechnol Prog 19:45; Doran (2013) §10.11. kLa_CO2/kLa_O2 ratio: Royce (1992) Biotechnol Bioeng 40:1129.

⚠  kLa_CO2 / kLa_O2 = 0.9 is a fixed empirical constant in correlations.ts. The UI calculation breakdown labels it as √(D_CO2/D_O2), which would give ~0.957 from diffusivities — not 0.9. The 0.9 value is a coarser empirical observation incorporating effects beyond diffusion (e.g., reaction-enhanced absorption); the label and constant should agree.

⚠  CO₂ score is forced to "low" when CDW < 20 g/L AND OUR < 30 mmol/L/h. The activation gate prevents reporting noisy results at low cell density, but a smoother fade-in or always-on calculation would be more defensible. Some low-density high-OUR scenarios may genuinely have non-negligible pCO₂.

⚠  P. pastoris always uses methanol RQ = 0.67 in resolveRq, regardless of the actual growth phase. The kinetic params in the same engine are for the glycerol batch phase (qO2 5 mmol/g/h, RQ ≈ 1.0). For glycerol-phase assessments, CER is under-estimated by ~33%.

⚠  Hydrostatic re-scaling: pCO₂_bulk uses an average column pressure, then pCO₂_bottom multiplies by the bottom-vs-atm pressure ratio. This double-counts a fraction of the pressure dependence. The correct chain is: equilibrium dissolved concentration scales with local gas-phase pressure, evaluated at the bottom. Effect is modest (~5–10% for typical H_L) but the model is internally inconsistent.

10. R5 — Heat removal risk

Margin = jacket cooling capacity / metabolic heat. Margin < 1 → cooling insufficient.

Q_metabolic [kW] = coeff · OUR · V_L / 3600

Metabolic heat coefficient (kJ / mmol O₂): bacteria & yeast 0.46; P. pastoris 0.52.

Ref: Cooney, Wang & Mateles (1968) Biotechnol Bioeng 11:269. P. pastoris methanol value from Roels (1983) Energetics and Kinetics in Biotechnology.

Cooling-water outlet temperature (jacket-side energy balance):

ṁ_cw  = (flow_lpm / 60) · 999 / 1000        (kg/s)

ΔT_cw = Q · 1000 / (ṁ_cw · 4184)            (K)

T_cw_out = T_cw_in + ΔT_cw

Log-mean ΔT across jacket (broth at uniform T_proc):

ΔT_1 = T_proc − T_cw_in     ΔT_2 = T_proc − T_cw_out

LMTD = (ΔT_1 − ΔT_2) / ln(ΔT_1 / ΔT_2)

Broth-side film coefficient (Chilton-Drew with viscosity correction set to 1):

Re_imp = ρ · N · d_imp² / μ

Pr     = Cp_broth · μ / k_broth        Cp_broth 4180 J/kg·K · k_broth 0.600 W/m·K

Nu     = C · Re_imp^(2/3) · Pr^(1/3)

h_i    = Nu · k_broth / T

Chilton-Drew constant C: Rushton 0.74 · Pitched-blade 0.55 · Marine 0.54 · Unknown 0.74.

Ref: Chilton & Drew (1944) Trans AIChE 40:155; Bondy & Lippa (1983) for PBT/marine.

Jacket-side film coefficient (Dittus-Boelter, water heated by warm broth):

gap   = clamp(0.015 · D_T, 5 mm, 25 mm)

D_h   = 2 · gap          A_c = 0.8 · π · gap · (D_T + gap)

u_jkt = F / A_c          Re_j = ρ_w · u_jkt · D_h / μ_w

Nu    = 0.023 · Re_j^0.8 · Pr_w^0.4         Re_j ≥ 10000

Nu    = transitional or 3.66 floor          otherwise

h_o   = Nu · k_w / D_h     (with hard floor 75 W/m²·K)

Ref: Dittus & Boelter (1930) Univ Calif Publ Eng 2:443; Incropera & DeWitt (2007) Fundamentals of Heat and Mass Transfer.

Overall U via series resistances (broth + fouling + wall + fouling + jacket):

1/U = 1/h_i + R_f_broth + δ_wall/k_wall + R_f_jacket + 1/h_o

R_f_broth = 3e-4 m²·K/W      R_f_jacket = 2e-4 m²·K/W

Wall: glass 1.1 W/m·K × 6 mm for V ≤ 10 L; SS316 16.0 W/m·K × 5 mm otherwise. U is clamped to [100, 180 W/m²·K] for glass and [100, 400 W/m²·K] for SS316.

Jacket area (cylindrical side + bottom disc):

A_total = π·D_T·H_L + π·D_T²/4

Cooling capacity and margin:

Q_cool_max = U · A_total · LMTD / 1000      (kW)

margin     = Q_cool_max / Q_metabolic

Score thresholds on the margin:

low > 1/0.60 ≈ 1.67 · moderate > 1/0.85 ≈ 1.18 · high > 1.0 · critical otherwise

Ref: Margin bands implied by HEAT_THRESHOLDS = {0.60, 0.85, 1.0} on Q_met/Q_cool form.

⚠  The viscosity correction (μ_broth / μ_wall)^0.14 in Chilton-Drew is set to 1.0 in deriveBrothFilmCoeff because wall temperature is unknown without iteration. This biases h_i in either direction depending on whether the wall is hotter or colder than the bulk; effect is typically ±10–15%.

⚠  JACKET_EFFECTIVE_AREA_FRACTION = 0.80 is an engineering approximation, not a universal constant. Vessels with half-pipe or dimple jackets have different effective flow areas. The constant directly scales u_jkt and Re_j.

⚠  U is clamped to [100, 400 W/m²·K] (SS316) regardless of the correlation result. This prevents idealised film + clean-wall calculations from overpredicting U, but also hides legitimate variation. Glass cap at 180 W/m²·K reflects wall-limited heat transfer.

⚠  Bottom disc area is added to the side-wall area as if it were jacketed. Many small-scale bioreactors are not jacketed on the bottom (dished head with no jacket). This can overestimate A_total by 15–30% at small scale.

11. Primary bottleneck selection

Primary bottleneck is the domain with the worst score. Ties between domains at the same severity are broken by normalised "risk pressure":

OTR    : pressure = band_lower_bound / ratio

Mixing : pressure = band_lower_bound / ratio

Shear  : pressure = ratio / band_upper_bound

CO₂    : pressure = pCO2_bottom / pCO2_critical  (raw, not band-normalised)

Heat   : pressure = (1/band_threshold) / margin

All low → no bottleneck.

⚠  CO₂ normalisation uses the raw bottom/critical ratio rather than dividing by the band boundary the way the other domains do. This makes CO₂ tie-break behaviour inconsistent with OTR / mixing / shear / heat. In practice CO₂ rarely ties at critical, so the impact is small.

12. What-if analysis

The user can toggle modifications (operational and design); the active set is applied to a copy of ProcessInputs and runAssessment is re-run. Modifications:

Switch impeller (Rushton / Pitched-blade).

Increase RPM → clamp at scale max.

Decrease RPM → bisection until target RPM drops by 50, clamped at scale min.

Increase aeration → VVM clamp at scale max.

Increase oxygen saturation → sets o2_inlet from a slider.

Increase impeller diameter → d/T_target += 0.1 (cap 0.8). Sets target_rpm_override to the baseline target RPM, so the larger impeller spins at the same RPM and delivers higher Pg/V and kLa.

Decrease impeller diameter → d/T_target −= 0.1 (floor 0.1). No RPM override, so the criterion solver compensates with higher RPM, increasing tip speed.

Add internal cooling coils → Q_cool_max × 1.5 (post-hoc patch).

The Conflicts table prevents mutually inconsistent toggles (e.g., increase + decrease RPM).

⚠  Without the target_rpm_override path added for "Increase impeller diameter", the scale-up criterion solver would have absorbed the geometry change by lowering RPM, leaving kLa unchanged. This was fixed; the patch is in lib/engine/reactor_configs.ts (see also lib/types/index.ts: target_rpm_override).

⚠  "Add internal cooling coils" multiplies Q_cool_max by a fixed 1.5×. This is a single empirical bump rather than a recalculation based on coil geometry and additional area. The number is plausible but not derived.

13. Reference: physical and equipment constants

Physical:

ρ (broth)             = 1000 kg/m³

g                     = 9.81 m/s²

P_atm                 = 101325 Pa

H_CO2 (Henry, ~30 °C) = 0.034 mol/L/atm

O₂ ref mole fraction  = 0.209  (dry air)

Correlation constants:

VANT_RIET coalescing   : (0.026, 0.4, 0.5)

VANT_RIET non-coalesc. : (0.002, 0.7, 0.2)

RUSZKOWSKI_CONSTANT    : 5.9

KLA_CO2_O2_RATIO       : 0.9

METABOLIC_HEAT_FACTOR  : 0.46 kJ / mmol O₂  (default)

Equipment (impeller):

Rushton       : Np 5.0 ,  Pg/P 0.60 , d/T 0.33

Pitched-blade : Np 1.5 ,  Pg/P 0.80 , d/T 0.33

Marine        : Np 0.35,  Pg/P 0.85 , d/T 0.40

Unknown       : Np 5.0 ,  Pg/P 0.60 , d/T 0.33      (mapped to Rushton)

Validity flags:

VVM valid 0.3–2.0       (Pg/P correlation)

P/V sanity 500–8000 W/m³ (D3 flag)

14. Confidence reporting

Each risk reports a confidence level depending on whether OUR is measured:

OTR: high_confidence with measured OUR, directional otherwise.

Mixing: reliable with measured OUR, directional otherwise.

Shear: always reliable (no OUR dependence).

CO₂: reliable with measured OUR; explicitly labelled "widest confidence interval of the five domains" in either case.

Heat: reliable with measured OUR, directional otherwise.

Notes for review

⚠  Items flagged in red across this document should be reviewed against the dev spec (/docs/lemnisca_dev_spec.md). The most consequential are: (a) the multi-impeller spacing bug in gassed_power.ts, (b) the P. pastoris RQ inconsistency in co2_risk.ts, (c) the kLa_CO2 constant vs labelled formula mismatch, (d) the CO₂ hydrostatic double-correction, and (e) the score-from-moderate-scenario-only behaviour in OTR.