Methodology — Fermentation Scale-Up Risk Predictor

How the application projects lab-scale operation to target scale and grades engineering risk

The application takes lab-scale fermentation data and projects the target-vessel operating point (RPM, VVM, P/V, kLa, tip speed) under one of three classical scale-up criteria. It then evaluates five engineering risk domains using literature correlations and organism-specific thresholds. Inputs are clamped against scale-dependent equipment envelopes (1 L to ≥10,000 L) before any risk is reported.

Scale-up criteria — what is held constant from lab to target

Constant P/V (default). Holds gassed power per volume Pg/V constant. The target RPM is found by bisection until gassed power on the larger geometry yields the same Pg/V as the lab. Preserves average specific energy dissipation, which sets kLa for a given broth and aeration. Standard choice for aerobic microbial fermentations.

Constant kLa. Solves target RPM so the achievable kLa at the target scale matches the lab kLa. Used when oxygen transfer is the governing constraint and Pg/V is a poor proxy for kLa across geometry changes.

Constant tip speed (π·N·d_imp). Preserves maximum mechanical shear in the impeller swept zone. Appropriate for shear-sensitive systems (filamentous fungi, mammalian cells, fragile morphologies). Trades mixing intensity and OTR for cell integrity.

Risk domains — metric, justification, correlations, thresholds

OTR (oxygen transfer). Metric: kLa ratio = achievable / required, where required = OUR / (C* − C_L). Compares supply against demand at peak metabolic rate. kLa is computed as an ensemble of correlations selected by impeller count and broth rheology — van't Riet (1979), Linek (1987, 2004), Moucha (2003) for multi-impeller, Garcia-Ochoa & Gomez (2004) viscosity-corrected, Zhu (2001) for non-Newtonian broth — using gassed power from Michel-Miller (1962, Rushton/Smith) or Cui (1996, pitched-blade). Driving force C* − C_L follows Henry's law with hydrostatic head correction. Thresholds: 1.5 / 1.0 / 0.7 → low / moderate / high.

Mixing. Metric: t_O2 / θ_mix, the ratio of how long the dissolved-oxygen reservoir at C_L can sustain the local OUR to how long it takes for the impeller to circulate the vessel. A ratio above 10 means gradients dissipate before they form; below 1 indicates DO-depleted zones with overflow metabolism. θ_mix uses the Ruszkowski (1994) correlation, θ ∝ T² / (ε^(1/3)·d_imp^(4/3)), validated for H/D ∈ [0.8, 1.5]. Thresholds: 10 / 1 / 0.1.

Shear. Metric: tip speed ratio v_tip / v_threshold. v_tip = π·N·d_imp is the controlling variable for the Kolmogorov microscale at typical industrial energy dissipation, which is set by conditions in the swept zone. Thresholds (per organism): 8 m/s E. coli, 7 m/s B. subtilis, 5 m/s S. cerevisiae, 4 m/s P. pastoris. Score boundaries on the ratio: 0.7 / 1.0 / 1.3.

CO₂ accumulation. Metric: pCO₂_critical / pCO₂_bottom (worst case is the vessel bottom, where hydrostatic pressure raises partial pressure). pCO₂ comes from a steady-state gas-phase mass balance with stripping rate kLa_CO2 = 0.9·kLa_O2 and Henry's coefficient for CO₂ in water at process temperature. Critical pCO₂ per organism is from Mostafa & Gu (2003) and Doran (2013): 0.10 bar (yeast), 0.15 bar (B. subtilis, P. pastoris), 0.20 bar (E. coli). Thresholds on the margin: 1.5 / 1.0 / 0.75.

Heat removal. Metric: Q_cool_max / Q_metabolic. Metabolic heat = OUR × 0.46 kJ/mmol O₂. Cooling capacity is computed from the jacketed-vessel heat-transfer chain: broth-side coefficient h_i from Chilton-Drew (Nu = C·Re_imp^(2/3)·Pr^(1/3)); jacket-side h_j from Dittus-Boelter; LMTD from inlet/outlet jacket temperatures; overall U combines both films plus wall conduction and fouling resistances. Thresholds on the margin: 1/0.60 / 1/0.85 / 1/1.0.

Confidence and limits

Each domain reports a confidence level. When OUR is user-measured rather than estimated from organism kinetics literature, OTR, mixing, and heat upgrade to high-confidence; CO₂ remains directional even with measured OUR because its gas-phase model is a simplified plug-flow approximation. The Ruszkowski mixing correlation is flagged as carrying additional uncertainty for H/D > 1.5. All hardcoded constants and threshold bands are drawn from the development specification and the cited primary literature.